| SPECIFICATION （规格书） HN MODEL: DCDC2500W 主变压器 拟 制 PREPARED; 潘婷婷; 审 核 CHECK; 批 准 APPROVAL. 2022.10.09. APPROA SUPPLIER. |
| --- |
|  | SPECIFICATION CNTROL DRAWING | MODEL NAME: DCDC2500 电感 | REV | SHEET |
|  | HOONEON | 211020120600011 | A.00 | 1 |

| 1. DIMENSION & SCHEMATIC(UNIT:mm) 外型图和原理图 4.3+/_0.3 Note: - 繞線需平整﹐不可有线打結、刮伤等. - PIN1,2BASE以上部分之出入線需要全部上透明鐵氟龍套管且必須將絕緣部分直接插 - 入BASE孔內與BASE底部基本平齊﹐以防止耐壓不良. - 3. 成品外上UL熱縮套管﹐頂部套管需高出COIL1.0mm MIN. 4. 粗铜线須先剝皮再焊錫，焊錫深度不可深入BASE孔深度的1/3; 5. 焊錫須使用低溫度焊錫，除剝皮或焊錫部分外，其他部位絕緣層不可受損（熔損，刺破等)。 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | SPECIFICATION CNTROL DRAWING | MODEL NAME: DCDC2500 电感 | REV | SHEET |
|  | HOONEON | 211020120600011 | A.00 | 3 |

| 2. WINDING SPECIFICATION 绕线规格 WDG; WIRE MATERIAL; TURNS; ST; FIN; NOTE; INSULATION. N1; Ø0.2 *1 UEW-N; 200.0; 4; 3. N2; Ø1.6*1 EIW-H; 1.0; 1; 2. 3. ELECTRICAL REQUIREMENTS 电气规格要求 電氣特性：; PIN位; 測試條件. L; 163.2+/_35% mH; 4--3; 10KHZ,0.05V. DCR; 2.10 ohm; 4--3; 25℃. HI-POT; AC 2.0KV 3mA 3sec; coil-coil. |
| --- |
|  |  | SPECIFICATION CNTROL DRAWING |  | MODEL NAME: DCDC2500 电感 |  | REV |  | SHEET |  |
|  |  | HOONEON |  | 211020120600011 |  | A.00 |  | 6 |  |
| 4. MATERIAL LIST 材料清单 NO; NAME (名称); MODEL （型号）; MANUFACTURER (制造商); UL FILE N （安全认证号）. 1; CORE (磁芯); T10*6*4C R10K or Equivalent; VAKOS. 2; MAGNET WIRE (漆包线); UEW-N(130℃) or Equivalent; TAI-I ELECTRIC WIRE & CABLE CO., LTD. HENG YA ELECTRIC (DONGGUAN) LTD; E85640 E197768. 3; TRIPLE WIRE (绝缘线）; EIW-H(180℃） or Equivalent; CHENG DU SOUTH-WEST ELECTRIC LTD CO; E178366. 4; BASE (底座）; T375HF or Equivalent; CHANG CHUN PLASTICS CO LTD.; E59481. 5; EPOXY (环氧树脂）; E-500 or Equivalent; DONGGUAN EATTO ELECTRONIC MATERIAL CO LTD. 6; UL TUBE （热缩套管）; CB-HFT or Equivalent; CHANGYUAN ELECTRONICS GROUP CO LTD. 7; TFL TUBE （铁氟龙套管）; CYG-ZHP or Equivalent; CHANGYUAN ELECTRONICS GROUP CO LTD; E180908. 5. OTHER REQUIREMENTS 5.1 OPERATING TEMPERATURE Operating temperature 130℃ MAX. |
|  |  | SPECIFICATION CNTROL DRAWING |  | MODEL NAME: DCDC2500 电感 |  | REV |  | SHEET |  |
|  |  | HOONEON |  | 211020120600011 |  | A.00 |  | 6 |  |